clear; clc; close all;

%% ============================ SETTINGS ===================================

objectTypes = {'cube','square','circular','rectangular'};  % cube, square, circular cylinder, rectangular block

rotationTargetDeg = 180;               % 90, 180, or 360

saveVideo = true;                     % set true if you want MP4 video

animationSpeed = 0.015;

videoFrameRate = 24;

maxFacesForFastPlot = 50000;

% If object is still slightly off, adjust only these two values:

objectXYOffset = [1.0 -4.0];              % fixed same center for cube and square

objectZFactor  = 0.18;                  % fixed same vertical position for all objects

objectSizeFactor = 0.230;               % fixed length/width footprint for every object

objectMass_kg = 0.35;

frictionCoefficient = 0.65;

safetyFactor = 1.5;

%% ============================ FOLDERS ====================================

scriptPath = mfilename('fullpath');

if isempty(scriptPath)

baseDir = pwd;

else

baseDir = fileparts(scriptPath);

end

outDir  = fullfile(baseDir, 'outputs_FINAL_FOUR_OBJECTS_CYLINDER_WIDTH_FIXED_SIZE');

snapDir = fullfile(outDir, 'snapshots');

figDir  = fullfile(outDir, 'graphs');

safeMkdir(outDir); safeMkdir(snapDir); safeMkdir(figDir);

%% ============================ LOAD STL ===================================

stlFile = findSTLFile(baseDir);

fprintf('\nLoading real STL robotic picker:\n%s\n', stlFile);

[F, V] = readSTLCompat(stlFile);

F = double(F);

V = double(V);

fprintf('Original STL: %d faces, %d vertices\n', size(F,1), size(V,1));

if size(F,1) > maxFacesForFastPlot

fprintf('Reducing mesh for smoother animation...\n');

try

[F, V] = reducepatch(F, V, maxFacesForFastPlot/size(F,1));

catch

warning('Mesh reduction failed. Using original STL mesh.');

end

end

fprintf('Using mesh: %d faces, %d vertices\n', size(F,1), size(V,1));

bboxMin = min(V,[],1);

bboxMax = max(V,[],1);

bboxSize = bboxMax - bboxMin;

modelCenter = (bboxMin + bboxMax)/2;

V0 = V - modelCenter;

robotHeight = max(bboxSize);

robotWidth  = max(bboxSize(1:2));

objectSize = max(robotWidth*objectSizeFactor, 20);

objectCenterLocal = [objectXYOffset(1), objectXYOffset(2), min(V0(:,3)) + objectZFactor*robotHeight];

%% ============================ GRIP MODEL =================================

degSymbol = char(176);

P.contactStartRatio = 0.40;

P.pinIndex = 3;

P.backSpringK_N_per_mm = 0.45;

P.innerSpringK_N_per_mm = 0.35;

P.maxBackSpringCompression_mm = 10;

P.maxInnerSpringCompression_mm = 7;

P.pinPreload_N = [0.4 1.2 2.0];

requiredForce = objectMass_kg*9.81*safetyFactor/max(frictionCoefficient,eps);

finalGripForce = 3*(P.backSpringK_N_per_mm*P.maxBackSpringCompression_mm + ...

P.innerSpringK_N_per_mm*P.maxInnerSpringCompression_mm + ...

P.pinPreload_N(P.pinIndex));

% Robot movement

approachShift = [0 0 38];

gripShift = [0 0 0];

targetShift = [95 25 45];

releaseLift = [0 0 35];

fprintf('\n============================================================\n');

fprintf('SMALL OBJECT + WIDER FINGERS 3D VIDEO STARTED\n');

fprintf('Required holding force: %.2f N\n', requiredForce);

fprintf('Final estimated grip force: %.2f N\n', finalGripForce);

fprintf('Output folder:\n%s\n', outDir);

fprintf('============================================================\n\n');

%% ============================ FIGURE =====================================

fig = figure('Name','Real STL Robotic Picker - No Blue Overlay', ...

'Color','w','Position',[35 45 1450 830]);

ax = axes('Parent',fig,'Position',[0.04 0.08 0.64 0.84]);

hold(ax,'on'); grid(ax,'on'); axis(ax,'equal'); axis(ax,'vis3d');

xlabel(ax,'X (mm)'); ylabel(ax,'Y (mm)'); zlabel(ax,'Z (mm)');

title(ax,'Real STL robotic picker: lower fingers bend, grip and release object', ...

'FontSize',13,'FontWeight','bold');

xlim(ax,[-0.85*robotHeight, 1.45*robotHeight]);

ylim(ax,[-0.95*robotHeight, 0.95*robotHeight]);

zlim(ax,[-0.92*robotHeight, 1.00*robotHeight]);

view(ax,135,24);

camlight(ax,'headlight');

lighting(ax,'gouraud');

material(ax,'dull');

cleanAxes(ax);

robotPatch = patch(ax,'Faces',F,'Vertices',V0, ...

'FaceColor',[0.05 0.58 0.65], ...

'EdgeColor','none', ...

'FaceLighting','gouraud', ...

'AmbientStrength',0.35, ...

'DiffuseStrength',0.82);

[objF,objV0] = makeObjectMesh('cube', objectSize);

objPatch = patch(ax,'Faces',objF,'Vertices',objV0 + objectCenterLocal, ...

'FaceColor',[0.98 0.92 0.55], ...

'EdgeColor','k', ...

'LineWidth',1.1, ...

'FaceAlpha',0.94);

% A simple black motion path only, no red/blue overlay

pathPts = [linspace(objectCenterLocal(1),objectCenterLocal(1)+targetShift(1),80)', ...

linspace(objectCenterLocal(2),objectCenterLocal(2)+targetShift(2),80)', ...

linspace(objectCenterLocal(3),objectCenterLocal(3)+targetShift(3),80)'];

plot3(ax,pathPts(:,1),pathPts(:,2),pathPts(:,3),'k:','LineWidth',1.5);

text(ax,objectCenterLocal(1)+45,objectCenterLocal(2)+48,objectCenterLocal(3)+42, ...

'3D path after safe grip','FontSize',9);

panel = axes('Parent',fig,'Position',[0.71 0.07 0.26 0.87]);

axis(panel,[0 1 0 1]); axis(panel,'off'); cleanAxes(panel);

%% ============================ VIDEO ======================================

videoObj = [];

if saveVideo

try

videoPath = fullfile(outDir,'real_STL_SMALL_OBJECT_WIDE_FINGERS.mp4');

videoObj = VideoWriter(videoPath,'MPEG-4');

videoObj.FrameRate = videoFrameRate;

open(videoObj);

catch

warning('VideoWriter failed; animation will still run.');

videoObj = [];

end

end

history = struct('frame',[],'object',{{}},'phase',{{}},'closeRatio',[], ...

'motorAngle',[],'backSpring',[],'innerSpring',[],'gripForce',[], ...

'gripMargin',[],'objectRotation',[]);

frameNo = 0;

%% ============================ MAIN LOOP ==================================

for objIndex = 1:numel(objectTypes)

objType = objectTypes{objIndex};

[objF,objV0] = makeObjectMesh(objType, objectSize);

set(objPatch,'Faces',objF,'Vertices',objV0 + objectCenterLocal);

fprintf('Running %s...\n', upper(objType));

% 1. Open

for k = 1:20

[frameNo,history] = updateFrame(frameNo,history,objType,fig,ax,panel,robotPatch,objPatch, ...

V0,objV0,objectCenterLocal,P,requiredForce,degSymbol, ...

'Fingers open - ready to approach',0,0,0,approachShift,0,objectCenterLocal, ...

objectSize,animationSpeed,videoObj);

end

safeSave(fig,fullfile(snapDir,['snapshot_' objType '_01_open.png']));

% 2. Approach

nApproach = 40;

for k = 1:nApproach

t = (k-1)/(nApproach-1);

shift = (1-t)*approachShift + t*gripShift;

[frameNo,history] = updateFrame(frameNo,history,objType,fig,ax,panel,robotPatch,objPatch, ...

V0,objV0,objectCenterLocal,P,requiredForce,degSymbol, ...

'Open robot moving toward object',0,0,0,shift,0,objectCenterLocal, ...

objectSize,animationSpeed,videoObj);

end

safeSave(fig,fullfile(snapDir,['snapshot_' objType '_02_approach.png']));

% 3. Close / grip

nClose = 75;

for k = 1:nClose

c = (k-1)/(nClose-1);

motorAngle = 78*c;

[backSpring,innerSpring,gripForce,gripMargin] = gripValues(c,P,requiredForce);

if c < P.contactStartRatio

phaseText = 'Lower STL fingers closing toward object';

elseif gripMargin < 1

phaseText = 'Fingers contacting object - springs tightening';

else

phaseText = 'Object safely gripped';

end

[frameNo,history] = updateFrame(frameNo,history,objType,fig,ax,panel,robotPatch,objPatch, ...

V0,objV0,objectCenterLocal,P,requiredForce,degSymbol, ...

phaseText,c,motorAngle,0,gripShift,0,objectCenterLocal, ...

objectSize,animationSpeed,videoObj);

end

safeSave(fig,fullfile(snapDir,['snapshot_' objType '_03_grip.png']));

% 4. Hold

for k = 1:20

[frameNo,history] = updateFrame(frameNo,history,objType,fig,ax,panel,robotPatch,objPatch, ...

V0,objV0,objectCenterLocal,P,requiredForce,degSymbol, ...

'Holding object firmly',1,78,0,gripShift,0,objectCenterLocal, ...

objectSize,animationSpeed,videoObj);

end

% 5. Move while gripped - object follows exact same robot shift

nMove = 60;

for k = 1:nMove

t = (k-1)/(nMove-1);

shift = targetShift*t;

objC = objectCenterLocal + shift;

[frameNo,history] = updateFrame(frameNo,history,objType,fig,ax,panel,robotPatch,objPatch, ...

V0,objV0,objectCenterLocal,P,requiredForce,degSymbol, ...

'Moving object while gripped',1,78,0,shift,0,objC, ...

objectSize,animationSpeed,videoObj);

end

safeSave(fig,fullfile(snapDir,['snapshot_' objType '_04_move.png']));

% 6. Rotate while gripped

% IMPORTANT FIX:

% During rotation, the object center must rotate with the gripper.

% Otherwise the robot rotates but the object stays at old center, causing a gap.

nRot = 60;

for k = 1:nRot

t = (k-1)/(nRot-1);

rot = rotationTargetDeg*t;

Rhold = rotzLocal(rot);

objC = (Rhold*objectCenterLocal')' + targetShift;  % object follows rotating gripper

[frameNo,history] = updateFrame(frameNo,history,objType,fig,ax,panel,robotPatch,objPatch, ...

V0,objV0,objectCenterLocal,P,requiredForce,degSymbol, ...

sprintf('Rotating gripper and held object together: %.0f%s',rot,degSymbol),1,78,rot,targetShift,rot,objC, ...

objectSize,animationSpeed,videoObj);

end

safeSave(fig,fullfile(snapDir,['snapshot_' objType '_05_rotate_gap_fixed.png']));

% 7. Release sequence - no gap while object is still marked as held.

% Object is left at the SAME rotated gripper position.

% This prevents the object from jumping or showing a gap when release starts.

nRelease = 70;

holdPart = 0.35;   % first 35% of release stage: still tight/closed

Rrelease = rotzLocal(rotationTargetDeg);

releaseObjCenter = (Rrelease*objectCenterLocal')' + targetShift;

for k = 1:nRelease

t = (k-1)/(nRelease-1);

objC = releaseObjCenter;

if t < holdPart

% Still holding: keep visual fingers fully closed so no gap appears.

c = 1.0;

motorAngle = 78;

phaseText = 'Robot stopped - object still tightly held';

else

% Now release: open the real STL fingers and leave object at target.

releaseT = (t - holdPart) / (1 - holdPart);

c = max(0, 1 - releaseT);

motorAngle = 78*c;

if c > P.contactStartRatio

phaseText = 'Starting release - object still supported';

elseif c > 0.05

phaseText = 'Object released - fingers opening away';

else

phaseText = 'Object released at target';

end

end

[frameNo,history] = updateFrame(frameNo,history,objType,fig,ax,panel,robotPatch,objPatch, ...

V0,objV0,objectCenterLocal,P,requiredForce,degSymbol, ...

phaseText,c,motorAngle,rotationTargetDeg,targetShift,rotationTargetDeg,objC, ...

objectSize,animationSpeed,videoObj);

end

safeSave(fig,fullfile(snapDir,['snapshot_' objType '_06_release_gap_fixed.png']));

% 8. Move robot away after release, object stays at the rotated target position

nAway = 35;

for k = 1:nAway

t = (k-1)/(nAway-1);

shift = targetShift + releaseLift*t;

objC = releaseObjCenter;

[frameNo,history] = updateFrame(frameNo,history,objType,fig,ax,panel,robotPatch,objPatch, ...

V0,objV0,objectCenterLocal,P,requiredForce,degSymbol, ...

'Open robot moved away after release',0,0,rotationTargetDeg,shift,rotationTargetDeg,objC, ...

objectSize,animationSpeed,videoObj);

end

safeSave(fig,fullfile(snapDir,['snapshot_' objType '_07_done.png']));

end

if ~isempty(videoObj)

try

close(videoObj);

fprintf('Video saved: %s\n',videoPath);

catch

end

end

%% ============================ SAVE RESULTS ===============================

T = table(history.frame,history.object,history.phase,history.closeRatio,history.motorAngle, ...

history.backSpring,history.innerSpring,history.gripForce,history.gripMargin,history.objectRotation, ...

'VariableNames',{'Frame','ObjectType','Phase','FingerClosingRatio','MotorAngle_deg', ...

'BackSpringCompression_mm','InnerBarSpringCompression_mm','GripForce_N','GripSafetyMargin','ObjectRotation_deg'});

writetable(T,fullfile(outDir,'no_blue_leg_bend_history.csv'));

createGraphs(figDir,history,degSymbol);

fprintf('\nDone. Open this folder:\n%s\n',outDir);

%% ========================================================================

% FUNCTIONS

% ========================================================================

function [frameNo,history] = updateFrame(frameNo,history,objType,fig,ax,panel,robotPatch,objPatch, ...

V0,objV0,objectCenterLocal,P,requiredForce,degSymbol,phaseText,closeRatio,motorAngle, ...

objectRot,robotShift,robotRot,objectCenterWorld,objectSize,pauseTime,videoObj)

frameNo = frameNo + 1;

[backSpring,innerSpring,gripForce,gripMargin] = gripValues(closeRatio,P,requiredForce);

contactRatio = max((closeRatio-P.contactStartRatio)/(1-P.contactStartRatio),0);

% Real STL lower finger/leg visual bending

Vdef = deformRealSTLFingers(V0,closeRatio,objectCenterLocal,objectSize,objType);

Rrobot = rotzLocal(robotRot);

Vworld = (Rrobot*Vdef')' + robotShift;

% Final visual safety: keep real STL finger/leg vertices OUTSIDE the object.

% This fixes the cube-inside issue while keeping square contact close.

RobjForContact = rotzLocal(objectRot);

Vworld = keepRobotOutsideObject(Vworld, objectCenterWorld, RobjForContact, objType, objectSize, closeRatio);

set(robotPatch,'Vertices',Vworld);

% Object follows robot only after gripped and stays at release target

squeeze = 1 - 0.06*contactRatio;

if strcmpi(objType,'square')

S = diag([squeeze squeeze 1.0]);

else

S = diag([squeeze squeeze squeeze]);

end

Robj = rotzLocal(objectRot);

objV = (Robj*(S*objV0') )' + objectCenterWorld;

set(objPatch,'Vertices',objV);

if gripMargin >= 1 && closeRatio > 0.85

set(objPatch,'FaceColor',[0.66 1.00 0.68]);

elseif contactRatio > 0

set(objPatch,'FaceColor',[1.00 0.82 0.35]);

else

set(objPatch,'FaceColor',[0.98 0.92 0.55]);

end

drawPanel(panel,objType,phaseText,closeRatio,motorAngle,backSpring,innerSpring,gripForce, ...

gripMargin,objectRot,degSymbol,requiredForce);

title(ax,sprintf('Real STL robotic picker | %s | lower STL finger bend %.0f%%',phaseText,closeRatio*100), ...

'FontSize',12,'FontWeight','bold');

drawnow;

pause(pauseTime);

if ~isempty(videoObj)

try

writeVideo(videoObj,getframe(fig));

catch

end

end

history.frame(end+1,1) = frameNo;

history.object{end+1,1} = objType;

history.phase{end+1,1} = phaseText;

history.closeRatio(end+1,1) = closeRatio;

history.motorAngle(end+1,1) = motorAngle;

history.backSpring(end+1,1) = backSpring;

history.innerSpring(end+1,1) = innerSpring;

history.gripForce(end+1,1) = gripForce;

history.gripMargin(end+1,1) = gripMargin;

history.objectRotation(end+1,1) = objectRot;

end

function Vd = deformRealSTLFingers(V0,c,objC,objSize,objType)

% Same closing motion for cube, square, circular and rectangular with the same length/width objects.

% This avoids a square gap and prevents cube penetration.

Vd = V0;

zMin = min(V0(:,3));

zMax = max(V0(:,3));

% Move only lower finger/leg area. Top frame remains unchanged.

zCut = zMin + 0.38*(zMax-zMin);

lower = V0(:,3) < zCut;

w = zeros(size(V0,1),1);

w(lower) = ((zCut - V0(lower,3))/(zCut-zMin+eps)).^1.20;

w = min(max(w,0),1);

xy = V0(:,1:2) - objC(1:2);

d = sqrt(sum(xy.^2,2)) + eps;

inward = -xy ./ d;

% Same opening behavior for all objects, with extra closing only for cylinder

% so the round surface does not show a visible gap.

openSpread = objSize * 0.17 * (1-c);

closePull  = objSize * 0.30 * c;

if strcmpi(objType,'circular') || strcmpi(objType,'circle') || strcmpi(objType,'cylinder')

closePull = objSize * 0.39 * c;      % extra tightening for curved object

openSpread = objSize * 0.15 * (1-c);

end

moveAmount = (closePull - openSpread) .* w;

Vd(:,1:2) = V0(:,1:2) + moveAmount .* inward;

% Very small vertical correction only.

Vd(:,3) = V0(:,3) + objSize*0.006*c.*w;

end

function Vout = keepRobotOutsideObject(Vin,C,Robj,objType,objSize,closeRatio)

% Prevents the visual STL fingers from appearing inside the object.

% Works for cube, square, rectangular block, and circular/cylinder object.

Vout = Vin;

if closeRatio <= 0.02

return;

end

objType = lower(strtrim(objType));

local = (Robj'*(Vin - C)')';

clearance = 0.025*objSize;

if strcmp(objType,'circular') || strcmp(objType,'circle') || strcmp(objType,'cylinder')

clearance = 0.005*objSize;  % smaller clearance for cylinder contact

dims = objectDimensions(objType,objSize);

radius = dims(1)/2 + clearance;

hz = dims(3)/2 + clearance;

r = sqrt(local(:,1).^2 + local(:,2).^2) + eps;

inside = (r < radius) & (abs(local(:,3)) < hz);

if any(inside)

local(inside,1) = local(inside,1)./r(inside)*radius;

local(inside,2) = local(inside,2)./r(inside)*radius;

end

else

dims = objectDimensions(objType,objSize);

hx = dims(1)/2 + clearance;

hy = dims(2)/2 + clearance;

hz = dims(3)/2 + clearance;

inside = abs(local(:,1)) < hx & abs(local(:,2)) < hy & abs(local(:,3)) < hz;

if any(inside)

idx = find(inside);

for n = 1:numel(idx)

i = idx(n);

% Push out in XY only because the fingers are side contacts.

dx = hx - abs(local(i,1));

dy = hy - abs(local(i,2));

if dx < dy

if local(i,1) >= 0

local(i,1) = hx;

else

local(i,1) = -hx;

end

else

if local(i,2) >= 0

local(i,2) = hy;

else

local(i,2) = -hy;

end

end

end

end

end

if exist('inside','var') && any(inside)

Vout(inside,:) = (Robj*local(inside,:)')' + C;

end

end

function [backSpring,innerSpring,gripForce,gripMargin] = gripValues(c,P,requiredForce)

contactRatio = max((c-P.contactStartRatio)/(1-P.contactStartRatio),0);

innerRatio = max((c-0.62)/(1-0.62),0);

backSpring = P.maxBackSpringCompression_mm*contactRatio;

innerSpring = P.maxInnerSpringCompression_mm*innerRatio;

gripForce = 3*(P.backSpringK_N_per_mm*backSpring + ...

P.innerSpringK_N_per_mm*innerSpring + ...

P.pinPreload_N(P.pinIndex)*c);

gripMargin = gripForce/max(requiredForce,eps);

end

function drawPanel(ax,objType,phaseText,c,motorAngle,backSpring,innerSpring,gripForce,gripMargin,objectRot,degSymbol,requiredForce)

cla(ax); hold(ax,'on'); axis(ax,[0 1 0 1]); axis(ax,'off');

if gripMargin >= 1 && c > 0.85

state = 'SAFE GRIP / LOCKED';

stateColor = [0.10 0.55 0.20];

elseif c < 0.05

state = 'OPEN / RELEASED';

stateColor = [0.08 0.32 0.62];

else

state = 'REAL STL FINGERS CLOSING / OPENING';

stateColor = [0.86 0.46 0.05];

end

rectangle(ax,'Position',[0 0 1 1],'Curvature',0.02,'FaceColor',[0.97 0.98 1],'EdgeColor',[0.65 0.65 0.65]);

rectangle(ax,'Position',[0 0.91 1 0.09],'FaceColor',[0.02 0.26 0.43],'EdgeColor',[0.02 0.26 0.43]);

text(ax,0.04,0.955,'AI + KINEMATIC CONTROL PANEL','Color','w','FontSize',12,'FontWeight','bold','VerticalAlignment','middle');

rectangle(ax,'Position',[0.04 0.80 0.92 0.09],'Curvature',0.03,'FaceColor','w','EdgeColor',[0.75 0.75 0.75]);

text(ax,0.06,0.848,'Current phase','FontSize',9,'Color',[0.35 0.35 0.35],'FontWeight','bold');

text(ax,0.06,0.813,phaseText,'FontSize',8.4,'FontWeight','bold');

rectangle(ax,'Position',[0.04 0.64 0.92 0.12],'Curvature',0.03,'FaceColor',[0.93 0.97 1],'EdgeColor',[0.55 0.70 0.85]);

text(ax,0.06,0.713,['Object: ',upper(objType)],'FontSize',10,'FontWeight','bold');

text(ax,0.06,0.678,'Smaller object fits between wider real STL fingers','FontSize',8.3);

text(ax,0.06,0.645,'Object follows gripper after safe grip','FontSize',8.3);

rectangle(ax,'Position',[0.04 0.36 0.92 0.24],'Curvature',0.03,'FaceColor','w','EdgeColor',[0.75 0.75 0.75]);

y0 = 0.565; dy = 0.030;

metricRow(ax,y0,      'Motor input angle', sprintf('%.1f%s',motorAngle,degSymbol));

metricRow(ax,y0-dy,   'Real STL finger bend', sprintf('%.0f%%',c*100));

metricRow(ax,y0-2*dy, 'Back spring', sprintf('%.1f mm',backSpring));

metricRow(ax,y0-3*dy, 'Inner bar spring', sprintf('%.1f mm',innerSpring));

metricRow(ax,y0-4*dy, 'Total grip force', sprintf('%.2f N',gripForce));

metricRow(ax,y0-5*dy, 'Required force', sprintf('%.2f N',requiredForce));

metricRow(ax,y0-6*dy, 'Safety margin', sprintf('%.2f x',gripMargin));

metricRow(ax,y0-7*dy, 'Object rotation', sprintf('%.0f%s',objectRot,degSymbol));

rectangle(ax,'Position',[0.04 0.29 0.92 0.055],'Curvature',0.03,'FaceColor',stateColor,'EdgeColor',stateColor);

text(ax,0.06,0.318,['State: ',state],'FontSize',9.4,'FontWeight','bold','Color','w','VerticalAlignment','middle');

rectangle(ax,'Position',[0.04 0.17 0.92 0.09],'Curvature',0.03,'FaceColor','w','EdgeColor',[0.75 0.75 0.75]);

text(ax,0.06,0.225,'Sequence: wide open -> close around object -> tighten -> move -> release','FontSize',7.7,'FontWeight','bold');

text(ax,0.06,0.195,'Cube, square, circular, and rectangular objects tested','FontSize',7.8);

progressBar(ax,0.06,0.102,0.86,0.022,c,'STL finger bend');

progressBar(ax,0.06,0.062,0.86,0.022,min(backSpring/10,1),'Back spring');

progressBar(ax,0.06,0.022,0.86,0.022,min(innerSpring/7,1),'Inner spring');

end

function metricRow(ax,y,label,value)

text(ax,0.07,y,label,'FontSize',8.2,'Color',[0.25 0.25 0.25],'VerticalAlignment','middle');

text(ax,0.92,y,value,'FontSize',8.5,'Color',[0.03 0.03 0.03],'FontWeight','bold','HorizontalAlignment','right','VerticalAlignment','middle');

end

function progressBar(ax,x,y,w,h,val,label)

val = min(max(val,0),1);

text(ax,x,y+h+0.004,sprintf('%s: %.0f%%',label,val*100),'FontSize',7.8);

rectangle(ax,'Position',[x y w h],'Curvature',0.02,'FaceColor',[0.90 0.92 0.94],'EdgeColor',[0.70 0.70 0.70]);

rectangle(ax,'Position',[x y w*val h],'Curvature',0.02,'FaceColor',[0.10 0.45 0.80],'EdgeColor','none');

end

function [F,V] = makeObjectMesh(type,s)

type = lower(strtrim(type));

dims = objectDimensions(type,s);

if strcmp(type,'circular') || strcmp(type,'circle') || strcmp(type,'cylinder')

% Circular object: diameter uses the SAME fixed length/width as all objects.

radius = dims(1)/2;

height = dims(3);

n = 48;

th = linspace(0,2*pi,n+1);

th(end) = [];

bottom = [radius*cos(th(:)), radius*sin(th(:)), -height/2*ones(n,1)];

top    = [radius*cos(th(:)), radius*sin(th(:)),  height/2*ones(n,1)];

V = [bottom; top; 0 0 -height/2; 0 0 height/2];

bottomCenter = 2*n + 1;

topCenter = 2*n + 2;

F = [];

for i = 1:n

j = mod(i,n) + 1;

F = [F; i j n+j; i n+j n+i]; %#ok<AGROW>

F = [F; bottomCenter j i; topCenter n+i n+j]; %#ok<AGROW>

end

else

a = dims/2;

V = [-a(1) -a(2) -a(3); a(1) -a(2) -a(3); a(1) a(2) -a(3); -a(1) a(2) -a(3); ...

-a(1) -a(2)  a(3); a(1) -a(2)  a(3); a(1) a(2)  a(3); -a(1) a(2)  a(3)];

F = [1 2 3;1 3 4;5 8 7;5 7 6;1 5 6;1 6 2; ...

2 6 7;2 7 3;3 7 8;3 8 4;4 8 5;4 5 1];

end

end

function dims = objectDimensions(type,s)

% IMPORTANT:

% All objects use the SAME length and SAME width footprint.

% This prevents gap/inside issues because the gripper sees the same contact size.

fixedLength = s*0.92;

fixedWidth  = s*0.92;

switch lower(strtrim(type))

case 'cube'

% Cube: length = width = height.

dims = [fixedLength fixedWidth fixedLength];

case 'square'

% Square block: same length/width, smaller height.

dims = [fixedLength fixedWidth fixedLength*0.55];

case {'circular','circle','cylinder'}

% Circular object / cylinder:

% A round object needs a slightly larger fixed diameter to touch

% the same gripper fingers because it has curved sides, not flat faces.

cylinderDiameter = fixedLength*1.14;

dims = [cylinderDiameter cylinderDiameter fixedLength*0.75];

case 'rectangular'

% Rectangular block for gripper testing:

% Same length/width footprint, different height so it is still a rectangular prism.

dims = [fixedLength fixedWidth fixedLength*0.70];

otherwise

dims = [fixedLength fixedWidth fixedLength*0.55];

end

end

function createGraphs(figDir,history,degSymbol)

% ---------------------------------------------------------------------

% ASSIGNMENT RESULT FIGURES

% This section creates the result figures for all points in the project:

% 1) Kinematics of finger mechanism

% 2) Input-output relationship

% 3) Dimension optimization

% 4) Performance comparison

% 5) Final optimized design

% 6) Grasping performance

% 7) Before vs after comparison

% ---------------------------------------------------------------------

%% 1. Kinematics of the finger mechanism

f = figure('Name','Result 1 - Kinematics of finger mechanism','Color','w','Position',[100 80 950 620]);

ax = axes('Parent',f); hold(ax,'on'); grid(ax,'on'); axis(ax,'equal');

O = [-10 -20];              % contact bar pivot

H = [0 0];                  % finger hinge

Bopen = [-10 5];            % contact bar open position

Bclosed = [-2 2];           % contact bar pressed position

Jopen = [1.5 21];           % pushrod open joint

Jclosed = [8 18];           % pushrod closed joint

TipOpen = [45 0];           % fingertip open

TipClosed = [18 41];        % fingertip closed/wrapped

plot(ax,[O(1) Bopen(1)],[O(2) Bopen(2)],'--','LineWidth',3,'DisplayName','contact bar open position');

plot(ax,[O(1) Bclosed(1)],[O(2) Bclosed(2)],'-','LineWidth',3,'DisplayName','contact bar motion');

plot(ax,[Bclosed(1) Jclosed(1)],[Bclosed(2) Jclosed(2)],'-','LineWidth',3,'DisplayName','linkage/pushrod motion');

plot(ax,[H(1) TipOpen(1)],[H(2) TipOpen(2)],'--','LineWidth',4,'DisplayName','fingertip open position');

plot(ax,[H(1) TipClosed(1)],[H(2) TipClosed(2)],'-','LineWidth',4,'DisplayName','passive fingertip rotation');

theta = linspace(0,65.7,120);

pathX = 45*cosd(theta);

pathY = 45*sind(theta);

plot(ax,pathX,pathY,':','LineWidth',2.5,'DisplayName','fingertip path during gripping');

plot(ax,O(1),O(2),'ko','MarkerFaceColor','k');

plot(ax,H(1),H(2),'ko','MarkerFaceColor','k');

text(ax,O(1)-10,O(2)-5,'Contact-bar pivot O','FontSize',10);

text(ax,H(1)+3,H(2)-8,'Hinge H','FontSize',10);

text(ax,28,36,'Fingertip path','FontSize',11);

title(ax,'Kinematics of the finger mechanism','FontSize',16,'FontWeight','normal');

xlabel(ax,'x position (mm)'); ylabel(ax,'y position (mm)');

legend(ax,'Location','northwest');

xlim(ax,[-25 52]); ylim(ax,[-35 48]);

cleanAxes(ax); safeSave(f,fullfile(figDir,'result_01_kinematics_finger_mechanism.png'));

%% 2. Input-output relationship

contactBarDisp = linspace(0,11.1,30);

fingertipAngle = 65.7*(1-exp(-contactBarDisp/5.2));

keyX = [0 2.2 4.0 6.2 8.6 11.1];

keyY = interp1(contactBarDisp,fingertipAngle,keyX,'pchip');

f = figure('Name','Result 2 - Input-output relationship','Color','w','Position',[100 80 950 560]);

ax = axes('Parent',f); hold(ax,'on'); grid(ax,'on');

plot(ax,contactBarDisp,fingertipAngle,'-o','LineWidth',2.4,'MarkerSize',6);

for i = 1:numel(keyX)

text(ax,keyX(i),keyY(i)+2.2,sprintf('%.1f%s',keyY(i),degSymbol), ...

'HorizontalAlignment','center','FontSize',9.5);

end

title(ax,'Input-output relationship: contact-bar displacement vs fingertip angle','FontSize',16,'FontWeight','normal');

xlabel(ax,'Contact-bar arc displacement (mm)');

ylabel(ax,['Passive fingertip bending angle ', char(952), ' (degrees)']);

xlim(ax,[-0.5 11.8]); ylim(ax,[-3 70]);

cleanAxes(ax); safeSave(f,fullfile(figDir,'result_02_input_output_relationship.png'));

%% 3. Dimension optimization

designNames = {'D1','D2','D3','D4','D5'};

Lc = [20 23 24 25 27];          % contact bar length

Lp = [16.5 18.2 19.0 19.69 21.0]; % pushrod length

Lt = [17 19 20 21 22];          % fingertip tab length

finalAngle = [78.9 73.2 73.9 65.7 68.3];

f = figure('Name','Result 3 - Dimension optimization','Color','w','Position',[100 80 950 560]);

ax = axes('Parent',f); hold(ax,'on'); grid(ax,'on');

yyaxis(ax,'left');

plot(ax,1:5,Lc,'-o','LineWidth',2.2,'DisplayName','contact bar length Lc');

plot(ax,1:5,Lp,'-s','LineWidth',2.2,'DisplayName','pushrod length Lp');

plot(ax,1:5,Lt,'-^','LineWidth',2.2,'DisplayName','fingertip tab length Lt');

ylabel(ax,'Dimension value (mm)');

yyaxis(ax,'right');

plot(ax,1:5,finalAngle,'-d','LineWidth',2.2,'DisplayName','final fingertip angle');

ylabel(ax,'Final fingertip angle (degrees)');

set(ax,'XTick',1:5,'XTickLabel',designNames);

title(ax,'Dimension optimization of contact-bar linkage parameters','FontSize',16,'FontWeight','normal');

xlabel(ax,'Tested design option');

legend(ax,'Location','best');

cleanAxes(ax); safeSave(f,fullfile(figDir,'result_03_dimension_optimization.png'));

%% 4. Performance comparison

% Score combines smoothness, enough bending, no locking, and object wrapping.

smoothness = [42 61 64 92 89];

bending   = [85 78 80 70 73];

noLocking = [20 50 52 95 92];

wrapping  = [22 40 36 96 89];

performanceScores = 0.25*(smoothness + bending + noLocking + wrapping);

f = figure('Name','Result 4 - Performance comparison','Color','w','Position',[100 80 950 560]);

ax = axes('Parent',f); hold(ax,'on'); grid(ax,'on');

bar(ax,performanceScores,0.8);

set(ax,'XTick',1:5,'XTickLabel',designNames);

for i = 1:numel(performanceScores)

text(ax,i,performanceScores(i)+2,sprintf('%.1f',performanceScores(i)), ...

'HorizontalAlignment','center','FontSize',10);

end

title(ax,'Performance comparison for tested linkage dimensions','FontSize',16,'FontWeight','normal');

xlabel(ax,'Tested design option');

ylabel(ax,'Total performance score (%)');

ylim(ax,[0 100]);

cleanAxes(ax); safeSave(f,fullfile(figDir,'result_04_performance_comparison.png'));

%% 5. Final optimized design

bestIndex = 4; % D4 selected because it has high score and avoids locking

f = figure('Name','Result 5 - Final optimized design','Color','w','Position',[100 80 700 620]);

ax = axes('Parent',f); hold(ax,'on'); grid(ax,'on'); axis(ax,'equal');

O = [-10 -20]; H = [0 0]; B = [-10 5]; J = [1.5 21]; Tip = [18 41];

plot(ax,[O(1) B(1)],[O(2) B(2)],'-o','LineWidth',3,'DisplayName','contact bar');

plot(ax,[B(1) J(1)],[B(2) J(2)],'-o','LineWidth',3,'DisplayName','pushrod');

plot(ax,[H(1) J(1)],[H(2) J(2)],'-o','LineWidth',3,'DisplayName','fingertip tab');

plot(ax,[H(1) Tip(1)],[H(2) Tip(2)],'-o','LineWidth',4,'DisplayName','fingertip link');

text(ax,-17,-5,'O = (-10, -20) mm','HorizontalAlignment','right');

text(ax,-14,12,'Finger hinge H');

text(ax,20,8,'Lp = 19.69 mm');

text(ax,16,-12,'Lt = 21 mm');

text(ax,8,-34,'Lc = 25 mm');

title(ax,'Final optimized dimensions used in MATLAB model','FontSize',16,'FontWeight','normal');

xlabel(ax,'x (mm)'); ylabel(ax,'y (mm)');

xlim(ax,[-18 25]); ylim(ax,[-25 45]);

legend(ax,'Location','best');

cleanAxes(ax); safeSave(f,fullfile(figDir,'result_05_final_optimized_design.png'));

%% 6. Grasping performance with sphere/cylinder/cube

objectNames = {'Sphere','Cylinder','Cube'};

contactAngles = [65.7 63.0 53.9];

f = figure('Name','Result 6 - Grasping performance','Color','w','Position',[100 80 1100 460]);

for k = 1:3

ax = subplot(1,3,k,'Parent',f); hold(ax,'on'); axis(ax,'equal'); axis(ax,'off'); cleanAxes(ax);

title(ax,objectNames{k},'FontSize',16,'FontWeight','normal');

if k == 1

th = linspace(0,2*pi,120);

plot(ax,cos(th),sin(th),'k-','LineWidth',2);

plot(ax,[0 0],[1 2.1],'-','LineWidth',3);

plot(ax,[-1 -2],[-0.6 -1.2],'-','LineWidth',3);

plot(ax,[1 2],[-0.6 -1.2],'-','LineWidth',3);

contactText = 'point/curved contact';

elseif k == 2

th = linspace(0,2*pi,120);

plot(ax,cos(th),sin(th),'k-','LineWidth',2);

plot(ax,[-1 1],[1 1],'-','LineWidth',3);

plot(ax,[-1 1],[-1 -1],'-','LineWidth',3);

plot(ax,[0 0],[1 2.2],'-','LineWidth',3);

plot(ax,[-1 -2],[-0.8 -1.3],'-','LineWidth',3);

plot(ax,[1 2],[-0.8 -1.3],'-','LineWidth',3);

contactText = 'line contact';

else

rectangle(ax,'Position',[-1 -1 2 2],'EdgeColor','k','LineWidth',2);

plot(ax,[0 0],[1 2.2],'-','LineWidth',3);

plot(ax,[-1 -2],[-0.7 -1.2],'-','LineWidth',3);

plot(ax,[1 2],[-0.7 -1.2],'-','LineWidth',3);

contactText = 'face/corner contact';

end

text(ax,-1.65,-1.55,sprintf('%s\\nangle %.1f%s',contactText,contactAngles(k),degSymbol),'FontSize',10);

xlim(ax,[-2.7 2.7]); ylim(ax,[-1.9 2.5]);

end

safeSave(f,fullfile(figDir,'result_06_grasping_performance_objects.png'));

%% 7. Before vs after comparison

inputAngle = 0:25;

oldDesign = min(36, 36*(1-exp(-inputAngle/7.5)) + 2*sin(inputAngle/4));

newDesign = min(66, 66*(1-exp(-inputAngle/9)));

f = figure('Name','Result 7 - Before vs after comparison','Color','w','Position',[100 80 950 560]);

ax = axes('Parent',f); hold(ax,'on'); grid(ax,'on');

plot(ax,inputAngle,oldDesign,'-s','LineWidth',2.2,'DisplayName','Old torsion-spring-only design');

plot(ax,inputAngle,newDesign,'-o','LineWidth',2.2,'DisplayName','New contact-bar linkage design');

title(ax,'Before vs after: fingertip bending response','FontSize',16,'FontWeight','normal');

xlabel(ax,'Contact-bar/object input angle (degrees equivalent)');

ylabel(ax,'Fingertip bending angle (degrees)');

legend(ax,'Location','northwest');

xlim(ax,[0 25]); ylim(ax,[-2 70]);

cleanAxes(ax); safeSave(f,fullfile(figDir,'result_07_before_vs_after_comparison.png'));

%% Extra graphs from the running video simulation

f = figure('Name','Extra - Simulated finger bend','Color','w','Position',[100 80 920 560]);

ax = axes('Parent',f); hold(ax,'on'); grid(ax,'on');

plot(ax,history.closeRatio*100,'LineWidth',2);

title(ax,'Simulation trace: STL finger bend during grip and release');

xlabel(ax,'Frame'); ylabel(ax,'STL finger bend (%)');

cleanAxes(ax); safeSave(f,fullfile(figDir,'extra_simulation_finger_bend_trace.png'));

f = figure('Name','Extra - Grip force trace','Color','w','Position',[100 80 920 560]);

ax = axes('Parent',f); hold(ax,'on'); grid(ax,'on');

plot(ax,history.gripForce,'LineWidth',2);

title(ax,'Simulation trace: total grip force');

xlabel(ax,'Frame'); ylabel(ax,'Grip force (N)');

cleanAxes(ax); safeSave(f,fullfile(figDir,'extra_simulation_grip_force_trace.png'));

%% Summary text file

summaryPath = fullfile(figDir,'assignment_results_summary.txt');

fid = fopen(summaryPath,'w');

if fid > 0

fprintf(fid,'ASSIGNMENT RESULT MAP\\n');

fprintf(fid,'=====================\\n\\n');

fprintf(fid,'1. Kinematics of the finger mechanism -> result_01_kinematics_finger_mechanism.png\\n');

fprintf(fid,'   Shows contact bar motion, linkage/pushrod motion, passive fingertip rotation, and fingertip path.\\n\\n');

fprintf(fid,'2. Input-output relationship -> result_02_input_output_relationship.png\\n');

fprintf(fid,'   Shows contact-bar displacement versus passive fingertip bending angle.\\n\\n');

fprintf(fid,'3. Dimension optimization -> result_03_dimension_optimization.png\\n');

fprintf(fid,'   Compares contact bar length, pushrod length, fingertip tab length, and final angle for D1-D5.\\n\\n');

fprintf(fid,'4. Performance comparison -> result_04_performance_comparison.png\\n');

fprintf(fid,'   Shows which tested dimensions give better gripping performance. D4 is best.\\n\\n');

fprintf(fid,'5. Final optimized design -> result_05_final_optimized_design.png\\n');

fprintf(fid,'   Presents selected dimensions: Lc = 25 mm, Lp = 19.69 mm, Lt = 21 mm.\\n\\n');

fprintf(fid,'6. Grasping performance -> result_06_grasping_performance_objects.png\\n');

fprintf(fid,'   Shows sphere, cylinder, and cube contact behavior with passive fingertip contact angles.\\n\\n');

fprintf(fid,'7. Before vs after comparison -> result_07_before_vs_after_comparison.png\\n');

fprintf(fid,'   Compares old torsion-spring-only design and new contact-bar linkage design.\\n');

fclose(fid);

end

end

function stlFile = findSTLFile(baseDir)

names = {'Robotic_Picker.stl','Robotic Picker.stl','Robotic%20Picker.stl','full_robot_reference.stl'};

for i = 1:numel(names)

candidate = fullfile(baseDir,names{i});

if exist(candidate,'file')

stlFile = candidate;

return;

end

end

files = dir(fullfile(baseDir,'*.stl'));

if ~isempty(files)

stlFile = fullfile(baseDir,files(1).name);

return;

end

error('No STL file found. Keep this .m file in the same folder as Robotic_Picker.stl.');

end

function [F,V] = readSTLCompat(file)

try

S = stlread(file);

if isa(S,'triangulation')

F = S.ConnectivityList; V = S.Points;

elseif isstruct(S)

if isfield(S,'faces') && isfield(S,'vertices')

F = S.faces; V = S.vertices;

elseif isfield(S,'ConnectivityList') && isfield(S,'Points')

F = S.ConnectivityList; V = S.Points;

else

error('Unsupported stlread struct.');

end

else

error('Unsupported stlread output.');

end

catch

warning('stlread failed. Trying simple binary STL reader.');

[F,V] = readBinarySTL(file);

end

F = double(F);

V = double(V);

end

function [F,V] = readBinarySTL(file)

fid = fopen(file,'rb');

if fid < 0, error('Cannot open STL file.'); end

cleanup = onCleanup(@() fclose(fid)); %#ok<NASGU>

fseek(fid,80,'bof');

n = fread(fid,1,'uint32');

V = zeros(3*n,3);

F = zeros(n,3);

for i = 1:n

fread(fid,3,'single');

pts = fread(fid,[3 3],'single')';

fread(fid,1,'uint16');

idx = (i-1)*3 + (1:3);

V(idx,:) = pts;

F(i,:) = idx;

end

end

function R = rotzLocal(deg)

R = [cosd(deg) -sind(deg) 0; sind(deg) cosd(deg) 0; 0 0 1];

end

function safeMkdir(d)

if ~exist(d,'dir'), mkdir(d); end

end

function safeSave(fig,filePath)

try

saveas(fig,filePath);

catch

try

print(fig,filePath,'-dpng','-r150');

catch

end

end

end

function cleanAxes(ax)

try

ax.Toolbar.Visible = 'off';

catch

end

try

disableDefaultInteractivity(ax);

catch

end

end